# 2BHK Compact Modular Eco Estate

## Complete 2D CAD Drafting Brief — Reserva Verde Goa

Built-up: 2,500 sq.ft · Estate: 1 Acre · Ground-level only · 6 × 40 ft HC + 1 × 20 ft modules · U-shape courtyard plan

A premium ₹2000 Cr-level private eco-jungle estate concept for HNI buyers, NRIs, investors, wellness retreat users, and farm-stay owners. This document is the complete drafter handover for a 26-sheet CAD drawing package.

## 1. Design concept summary

A premium ground-level modular forest estate built from six 40 ft high-cube ISO containers and one 20 ft service module, arranged in a U-shape pavilion plan around a central open-to-sky biophilic courtyard. Strictly single-storey, strictly ground-level — no stacking, no vertical assembly, no hotel-pod typology. The architecture reads as a contemporary tropical forest pavilion — bamboo-batten rainscreen on container faces, charred-timber feature wall on the entry elevation, laterite stone base course at 300 mm plinth, lime-plaster end panels, standing-seam metal roof with deep eaves and integrated solar PV.

Design language references: Aman forest retreat × Six Senses eco-luxury × Goan jungle villa × modern modular architecture × forest-facing private estate. The home is sized and engineered for a couple or small family with optional rental / farm-stay readiness.

## 2. 2BHK 2,500 sq.ft planning logic

The 2,500 sq.ft is built from three layers stacked horizontally on a single ground plane:

| Layer | Area (sq.ft) | What sits here |
| --- | --- | --- |
| Container interiors (6 × 40 ft HC + 1 × 20 ft) | ~2,080 | Living, Dining, Kitchen, Master suite, BR2 suite, Study, Utility, Store, Powder |
| Insulated glass-link connectors | ~180 | Glass spine that ties the U together |
| Roofed verandah + courtyard-facing covered transition spaces | ~240 | South verandah + plantation-deck overlap + courtyard-edge sit-out |
| Total built-up | ~2,500 | ±2% architectural tolerance |

The U-shape opens to the south plantation, with the courtyard sitting at the geometric centre. Cross-ventilation pulls SW monsoon air through the glass spine; deep north-east eaves shelter the bedroom wing; the kitchen has a separate service entry hidden behind a hedged side yard.

## 3. Recommended layout type

Three layouts compared:

| Option | Pros | Cons | Score |
| --- | --- | --- | --- |
| 1. Linear Forest Villa | Simple construction; easy services; long forest-facing deck | Less premium identity; no internal courtyard | 6.5 |
| 2. L-Shape Forest Container Home | Better privacy; strong indoor-outdoor; good plantation-facing deck | No protected courtyard centre | 8.0 |
| 3. U-Shape Courtyard Container Home ✓ | Most premium estate identity; protected open-to-sky courtyard; strongest retreat feel; cross-ventilation excellent; rental-ready | Slightly higher construction logistics (mitigated by linear glass spine) | 9.2 |

Recommended: Option 3 — U-Shape Courtyard. The courtyard is the architectural and emotional centre of the home. It cools the building via stack effect, brings daylight into every habitable room, and gives the property a signature aerial read. For a 2BHK at 2,500 sq.ft the U-shape is the sweet spot: large enough for a courtyard, small enough to remain efficient and intimate.

## 4. 1-acre site zoning table

Total plot: 43,560 sq.ft (1 acre).

| Zone | Allocation (sq.ft) | % of plot | Notes |
| --- | --- | --- | --- |
| Entry, driveway, 2-car parking, arrival court | 2,800 | 6.4% | Gravel + paver, native canopy at gate |
| Home built-up | 2,500 | 5.7% | 6 × 40 ft + 1 × 20 ft |
| Covered outdoor (deck, verandah, pergola) | 1,000 | 2.3% | Excluded from built-up |
| Private lawn + garden + courtyard surround | 4,000 | 9.2% | Yoga lawn, firepit pad, meditation corner |
| Plantation zone | 26,000 | 59.7% | Buyer-selected profile |
| Herbal garden + food forest | 2,000 | 4.6% | Adjacent to kitchen |
| Water + utility + composting | 1,200 | 2.8% | RWH, recharge, bio-septic, reed bed |
| Walking trail + landscape buffer | 4,060 | 9.3% | Looped 0.8 km path |
| TOTAL | 43,560 | 100% | 1 acre |

## 5. Home placement options

| Option | Pros | Cons | Best for | Score |
| --- | --- | --- | --- | --- |
| A. Front-side | Easy access, max rear plantation, efficient construction | Less privacy, gate-to-home view too direct | First-phase buyers, easy build | 7.5 |
| B. Central courtyard placement ✓ | Premium estate arrival, balanced landscape on all 4 sides, strongest luxury feel | Slightly higher driveway cost | Recommended for 2BHK premium | 9.0 |
| C. Side placement | Consolidates plantation as one continuous block | Loses pavilion symmetry; one elevation faces driveway | Plantation-focused buyers | 7.5 |
| D. Rear placement | Max privacy, long luxury driveway, premium retreat feel | Higher infrastructure cost; long service runs | HNIs prioritising seclusion | 8.0 |

Recommended: Option B — Central placement for a 2BHK 2500 sq.ft model. The U-shape needs to be experienced from all four sides; central placement maximises this and gives every elevation a plantation backdrop. The arrival sequence is 40 ft from gate to forecourt, then 20 ft from forecourt to the entry foyer — short enough for a 2BHK, long enough for cinematic arrival.

## 6. Room-by-room area schedule (~2,500 sq.ft)

| # | Space | Area (sq.ft) | Module | Function & design notes |
| --- | --- | --- | --- | --- |
| 1 | Entrance Foyer | 100 | C-1 Part A (40 ft HC) | Pivot teak door 1050×2400 mm, slate tile, console + courtyard look-back |
| 2 | Living Room | 280 | C-1 Part B (40 ft HC) | Two-bay glazing, polished bamboo flooring, sliding south face |
| 3 | Dining Area | 200 | C-2 Part A (40 ft HC) | Mango-wood table seats 6, full courtyard glass wall |
| 4 | Open Kitchen | 200 | C-2 Part B (40 ft HC) | Granite counters, breakfast island, energy-star appliances |
| 5 | Master Bedroom | 300 | C-3 (40 ft HC) | South-east panoramic glazing, reading nook |
| 6 | Master Walk-in Wardrobe | 70 | C-4 Part A (40 ft HC) | Cedar-lined, dressing seat |
| 7 | Master Toilet | 140 | C-4 Part B (40 ft HC) | Indoor rain-shower wetroom, twin teak vanity, natural slate |
| 8 | Master Private Lounge nook | 110 | C-4 Part C (40 ft HC) | Reading corner with garden view, courtyard glass slider |
| 9 | Bedroom 2 | 280 | C-5 (40 ft HC) | NE window slot, writing desk alcove |
| 10 | Bedroom 2 Toilet | 110 | C-6 Part A (40 ft HC) | Wet zone, vanity, rain shower |
| 11 | Study / Family Lounge | 140 | C-6 Part B (40 ft HC) | Acoustic insulation, built-in desk + bookshelves |
| 12 | Powder Toilet | 50 | C-6 Part C (40 ft HC) | Off corridor, greywater-fed |
| 13 | Utility / Laundry | 90 | C-7 Part A (20 ft) | Heat-pump washer-dryer, solar hot-water manifold |
| 14 | Store / Pantry | 70 | C-7 Part B (20 ft) | Louvred ventilation, mosquito-screened |
| 15 | Glass-link Corridors (Links 1–7) | 180 | Insulated DGU spine | Primary circulation + ventilation |
| 16 | Covered Verandah | 200 | Roofed pergola | Reclaimed timber, bamboo sun screen |
| 17 | Courtyard-edge Transition | 100 | Open-to-sky surround | Stone-paver edge around the central courtyard |
| TOTAL APPROX. BUILT-UP |  | ~2,520 |  | ±2% architectural tolerance |

Outdoor (not counted in built-up):

| Space | Area | Notes |
| --- | --- | --- |
| Internal Courtyard (open-to-sky) | 260 | Stone water channel, native fern planting, single Plumeria tree |
| Optional plunge pool | 120 | 6 × 2.5 m, 600 mm deep, off the south deck |
| Outdoor dining + firepit pad | 200 | Pergola roof, stone seating |
| Yoga / meditation lawn | 800 | Soft-grass pad facing forest |

## 7. Container module strategy

Modules used: 6 × 40 ft HC + 1 × 20 ft service module = 7 containers total

| Module type | Size | Internal area | Use |
| --- | --- | --- | --- |
| 40 ft HC ISO container | 40' × 8' × 9.5' (12.19 × 2.44 × 2.90 m) | 320 sq.ft | Primary habitable volumes — living, dining, kitchen, master, BR2 |
| 20 ft ISO container | 20' × 8' × 9.5' (6.10 × 2.44 × 2.90 m) | 160 sq.ft | Service module — utility + store |
| Insulated DGU glass corridor | 2,200 mm × 3,000 mm × variable length | 180 sq.ft total | Glass spine linking modules |
| Roofed pergola verandah | Variable | 200 sq.ft | Covered south verandah |

How the 2,500 sq.ft is built up:

- 6 × 40 ft container interiors: 6 × 320 = 1,920 sq.ft

- 1 × 20 ft container interior: 160 sq.ft

- Glass connector corridors: 180 sq.ft (enclosed, climate-controlled)

- Enclosed verandah + courtyard transition: 240 sq.ft (roofed, partially enclosed)

- TOTAL BUILT-UP: ~2,500 sq.ft

Foundations: helical steel screw piles — zero concrete pour, mature tree-root preservation, 450 mm underfloor air gap for monsoon ventilation. Modules pre-fitted off-site (insulation + MEP + finishes) and craned into final position over 2 working days.

## 8. Container module table

| ID | Size | Wing | Program | Reinforcement / cutout strategy |
| --- | --- | --- | --- | --- |
| C-1 | 40' × 8' × 9.5' HC | South-west | Entry Foyer (100) + Living Room (280) | 28 ft south cutout. 150×150×6 mm MS square hollow columns at headers; pivot-door threshold detail |
| C-2 | 40' × 8' × 9.5' HC | South-east | Dining (200) + Open Kitchen (200) | 24 ft north cutout to courtyard. Heavy C-channel + flitch plates at kitchen island span; vented exhaust |
| C-3 | 40' × 8' × 9.5' HC | North-west | Master Bedroom (300) | 14 ft SE corner cutout. Flitch plates welded to corrugated sheet nodes |
| C-4 | 40' × 8' × 9.5' HC | North-west | Master WR (70) + Master Bath (140) + Master Lounge (110) | 12 ft cutout. Reinforced wet-zone framing, double-welded corner gussets, sealed wet-cell |
| C-5 | 40' × 8' × 9.5' HC | North-east | Bedroom 2 (280) | 14 ft east cutout. L-angle truss reinforcement at corners |
| C-6 | 40' × 8' × 9.5' HC | North-east | BR2 Bath (110) + Study/Family Lounge (140) + Powder (50) | 16 ft cutout. Sealed wet-cell + acoustic-insulated study walls |
| C-7 | 20' × 8' × 9.5' HC | East service yard | Utility/Laundry (90) + Store/Pantry (70) | No large cutouts. Reinforced pipe-penetration frames only |

How modules connect: All seven containers link through a continuous insulated glass-link spine running along the courtyard's inner edge. The spine is a 2,200 mm wide × 3,000 mm tall curtain-wall corridor with 1,200 mm mullion spacing, double-glazed low-E argon-filled units. Container-to-container joints are flashed and gasketed; floor levels unified with a 50 mm screed transition.

## 9. Complete CAD drawing sheet list (26 sheets)

| Sheet | Drawing | Scale | Sheet size |
| --- | --- | --- | --- |
| 01 | Cover Sheet + Drawing Index | NTS | A1 |
| 02 | 1-Acre Site Placement Plan | 1:200 | A1 |
| 03 | Site Circulation Plan | 1:200 | A1 |
| 04 | Detailed Floor Plan | 1:100 | A1 |
| 05 | Container Module Plan | 1:100 | A1 |
| 06 | Furniture Layout Plan | 1:100 | A1 |
| 07 | Dimension Plan | 1:100 | A1 |
| 08 | Floor Finish Plan | 1:100 | A1 |
| 09 | Reflected Ceiling Plan | 1:100 | A1 |
| 10 | Electrical Layout | 1:100 | A1 |
| 11 | Plumbing Layout | 1:100 | A1 |
| 12 | Sanitary Fixture Plan | 1:50 | A1 |
| 13 | Roof Plan | 1:100 | A1 |
| 14 | Front Elevation (South) | 1:100 | A1 |
| 15 | Rear Elevation (North) | 1:100 | A1 |
| 16 | Left Side Elevation (West) | 1:100 | A1 |
| 17 | Right Side Elevation (East) | 1:100 | A1 |
| 18 | Section A-A (longitudinal — living-courtyard-deck) | 1:50 | A1 |
| 19 | Section B-B (transverse — bedroom-bath-kitchen) | 1:50 | A1 |
| 20 | Door and Window Schedule | NTS | A1 |
| 21 | Material and Cladding Details | 1:20 / 1:10 | A1 |
| 22 | Structural Concept Plan | 1:100 | A1 |
| 23 | Landscape Concept Plan | 1:200 | A1 |
| 24 | Sustainability Plan | 1:100 / NTS | A1 |
| 25 | Future Expansion Plan | 1:100 | A1 |
| 26 | Area Statement | NTS | A1 |

## 10. Detailed instructions for each drawing sheet

### Sheet 01 — Cover Sheet

- Project name: Reserva Verde Goa

- Model: 2BHK Compact Modular Eco Estate · 2,500 sq.ft · 1 Acre

- Developer: Bluechip Global Agro

- Drawing index table (sheet number, title, scale, revision)

- Revision table (rev / date / description / drawn by / checked by)

- North arrow note

- Concept-stage disclaimer: "All drawings are concept-stage. Final design, dimensions, structural details, MEP, environmental compliance, and construction feasibility must be verified by licensed architects, structural engineers, surveyors, and relevant authorities."

### Sheet 02 — 1-Acre Site Placement Plan @ 1:200

Show on this sheet:

- 1-acre boundary (63,400 × 63,400 mm) with 12 ft landscape buffer

- South-edge entry gate (1,800 mm wide)

- Internal driveway (4 m wide, 12 m long, gravel + paver finish)

- Arrival forecourt (6 × 8 m, 2-car parking pad)

- Home footprint centrally placed (U-shape, 18 × 16 m envelope)

- Internal courtyard (8 × 5 m, open-to-sky)

- Covered verandah on south face

- Optional plunge pool (6 × 2.5 m) on east of south deck

- Private lawn + meditation pad (8 × 6 m)

- Plantation zone wrapping rear and sides (26,000 sq.ft)

- Herbal garden strip adjacent to kitchen (2,000 sq.ft)

- Walking trail loop (0.8 km, 900 mm wide stone-paver)

- RWH tank at NE corner (15,000 L, 2,500 mm Ø)

- Recharge pit (1,200 × 1,200 mm dashed square)

- Bio-septic zone (E side, 2,400 × 1,200 mm)

- Composting twin-chamber (1,200 × 800 mm) at utility yard

- Solar / inverter shed (2 × 2 m, SW corner)

- Future expansion zone outline (dashed) for third bedroom

- Privacy buffer (12 ft wide native canopy belt along boundary)

- North arrow (top-right), scale bar (bottom-left), legend (bottom-right)

- Title block (bottom-right)

### Sheet 03 — Site Circulation Plan @ 1:200

Overlay on Sheet 02:

- Vehicle entry path (gate → forecourt) — solid arrow

- Guest drop-off — circle marker at entry foyer

- Pedestrian path from forecourt to entry foyer (stone-paver)

- Service path from gate side yard to utility module C-7 (hedge-screened)

- Emergency / service access from boundary

- View corridor 1: forecourt → entry foyer → courtyard → far plantation (long axis sight line)

- View corridor 2: master bedroom → plantation south-east

- Privacy screening: hedge line along service path, bamboo screen on west elevation

### Sheet 04 — Detailed Floor Plan @ 1:100

Show:

- All 7 container module outlines (label C-1 to C-7)

- Internal partition walls (100 mm with insulation, 50 mm without)

- Wall thickness annotation

- Door swings (1/4-arc) annotated D-01 to D-14

- Window openings as gapped wall segments with sill line, annotated W-01 to W-12

- Sliding glass doors (SD-01 to SD-06) on south face and courtyard face

- Furniture layout (sofa cluster, dining table, beds, wardrobes — light strokes)

- Kitchen island + counter geometry

- Bathroom fixtures (WC, basin, shower tray, soaking nook)

- Wardrobe layout in master suite

- Utility module fittings

- Glass-link corridor outlines

- Central courtyard with stone water channel rectangle (3 × 1.2 m)

- Deck + verandah edge

- Service access path

- North arrow, scale bar, room labels, dimensions

### Sheet 05 — Container Module Plan @ 1:100

Diagrammatic plan showing:

- Container numbering: C-1 (Living) · C-2 (Dining + Kitchen) · C-3 (Master) · C-4 (Master Bath + WR + Lounge) · C-5 (BR2) · C-6 (BR2 Bath + Study + Powder) · C-7 (Utility + Store)

- 40 ft × 8 ft module outlines (12,200 × 2,440 mm) for C-1 to C-6

- 20 ft × 8 ft module outline (6,100 × 2,440 mm) for C-7

- Module orientation arrows (which face opens to courtyard)

- Glass-link connector corridor outlines between modules

- Roof canopy zones (dashed)

- Structural frame notes — corner castings, reinforcement strategy per container

- Schedule callout: "Modules pre-fitted off-site; lifted into position by 50T crane"

### Sheet 06 — Furniture Layout Plan @ 1:100

Show:

- Living room: 3-piece sofa cluster (3,000 × 800 mm sectional), 2 lounge chairs, coffee table (1,200 × 700 mm), TV console wall-mount, side tables

- Dining: 6-seater table (2,000 × 1,000 mm), 6 chairs, sideboard

- Kitchen: peninsula counter (2,400 × 900 mm), breakfast island (1,800 × 900 mm) with 3 stools

- Master bedroom: king bed (2,000 × 1,800 mm), 2 bedside tables, reading bench

- Master WR: walk-in cabinets, dressing seat (600 × 600 mm)

- BR2: queen bed (2,000 × 1,500 mm), wardrobe (2,400 × 600 mm), writing desk (1,500 × 600 mm)

- Study: writing desk (1,800 × 700 mm), task chair, bookshelves (2,400 × 350 mm)

- Outdoor deck: 4 loungers, outdoor dining table (1,800 × 800 mm), 6 chairs

- Poolside: 2 sun-loungers + side table (only if plunge pool added)

- All furniture in 0.5 pt grey strokes, dimensioned

### Sheet 07 — Dimension Plan @ 1:100

Show:

- Overall external dimensions of the home envelope (18 × 16 m or similar)

- Container module exterior dimensions (12.2 × 2.44 m per module)

- Room internal dimensions (every habitable room)

- Wall thicknesses (100 mm insulated, 50 mm partition)

- Door sizes (D-01 to D-14)

- Window sizes (W-01 to W-12)

- Sliding glass door dimensions (SD-01 to SD-06)

- Deck dimensions (verandah + courtyard surround)

- Courtyard dimensions (8 × 5 m)

- Setback notes from boundary (12 m on all sides typical)

- Finished floor level (FFL): +600 mm above natural ground level (allows underfloor ventilation)

### Sheet 08 — Floor Finish Plan @ 1:100

| Room | Finish | Specification |
| --- | --- | --- |
| Entrance foyer | Slate stone tile | 600 × 600 mm, dark grey, anti-skid |
| Living room | Engineered oak | 180 × 1,800 mm planks, oiled finish |
| Dining + Kitchen | Microcement | 4 mm seamless, light grey, sealed |
| Master bedroom | Engineered oak | 180 × 1,800 mm, oiled |
| Master toilet | Anti-skid stone tile | 300 × 300 mm, natural slate |
| BR2 + BR2 toilet | Microcement / anti-skid tile | Microcement bedroom, anti-skid in bath |
| Study / lounge | Engineered oak | 180 × 1,800 mm |
| Powder | Slate stone tile | Anti-skid |
| Utility | Vitrified tile | 600 × 600 mm, easy-clean |
| Store | Vitrified tile | 600 × 600 mm |
| Courtyard | Stone paver + gravel | 600 × 300 mm laterite paver + raked-gravel surround |
| Covered verandah | IPE / kapur timber deck | 145 mm planks, oiled |
| Outdoor deck / pool deck | IPE / WPC composite | 145 mm, oiled |
| Driveway | Permeable gravel + grass pavers | Laterite gravel + concrete grass-paver |
| Pathway | Laterite stone slab | 600 × 300 mm |

### Sheet 09 — Reflected Ceiling Plan @ 1:100

Show:

- Ceiling height note: 2,750 mm finished (from FFL); container height 2,896 mm clear with 150 mm service void

- False-ceiling zones: bathrooms (250 mm drop for plumbing), kitchen (300 mm drop for exhaust), entry foyer (300 mm cove)

- Cove lighting locations: foyer, living room perimeter, master bedroom head wall

- Downlight positions: 75 mm LED downlights, 4,000K kitchen / 3,000K living / 2,700K bedrooms

- Fan points: living (1), master (1), BR2 (1), study (1) — BLDC ceiling fans

- AC indoor unit positions: split-DX, one per habitable room (master, BR2, living-dining, study)

- Skylight option: above master toilet (1,200 × 800 mm tempered glass)

- Exposed beam option: living room ceiling — keep visible the recycled-steel I-beam at module junction for character

- Pendant light points: dining (1 × 800 mm cluster), entry (1 × 600 mm)

### Sheet 10 — Electrical Layout @ 1:100

Show on legend + plan:

- Light points: 75 mm LED downlights + cove + pendants + scones — total ~50 points

- Fan points: 4 BLDC ceiling fan points (living, master, BR2, study)

- AC points: 4 split-DX condenser points (concealed eastern service yard)

- Switch boards: at each entry, master headboard, BR2 headboard, study desk

- 5A sockets: typical wall positions — bedside (2 per BR), study desk (4), kitchen counter (4 × 5A USB), bathroom vanity (2)

- 15A / 16A sockets: AC dedicated (4), geyser (3), washer-dryer (1), microwave (1), refrigerator (1), induction hob (1), water pump (1)

- TV point: living room TV wall + master bedroom

- WiFi / router point: living room TV wall (provision for mesh node in master + study)

- Refrigerator point: dedicated 15A

- Microwave point: dedicated 15A, kitchen counter

- Hob / chimney point: 16A for induction; 5A for chimney

- Washing machine + dryer point: 15A, utility

- Geyser points: master bath (25L), BR2 bath (25L), kitchen (10L)

- Outdoor lights: verandah uplights (4), deck step lights (6), courtyard wall washers (6), path lights along walking trail (12)

- Garden lights: 5A weatherproof bollards (10 × 600 mm)

- DB (distribution board): utility wall, 24-way TPN

- Solar inverter location: SW utility shed, 5 kW hybrid inverter

- EV charging provision: 7.2 kW point at arrival forecourt

- Smart home provision: KNX or Loxone backbone, conduits for future automation

### Sheet 11 — Plumbing Layout @ 1:100

Show:

- Cold water lines (blue): from underground sump → booster pump → manifold → zoned distribution

- Hot water lines (red): from geyser to fixtures

- Kitchen sink inlet (15 mm) and outlet (50 mm) — connected to greywater line

- Bathroom plumbing: WC + basin + shower + tub (master only) — full detail

- WC soil line (orange): 100 mm PVC to bio-septic

- Shower drain: 75 mm trap with floor grating

- Vanity drain: 50 mm PVC to greywater line

- Utility drain: 75 mm to greywater + washer drainage to dedicated outlet

- Outdoor tap: 4 locations — garden hose bibs at corners

- Rainwater drainage: roof gutters → 100 mm downpipes → RWH tank

- Greywater reuse: kitchen + bath greywater → reed-bed wetland → drip irrigation

- Bio-septic / STP connection: 100 mm PVC to twin-chamber bio-septic at E utility zone

### Sheet 12 — Sanitary Fixture Plan @ 1:50

Per bathroom, show:

- WC: wall-hung dual-flush, 4.5/3 L, brand-equivalent Toto or Kohler

- Wash basin: counter-top stone basin (master) or wall-mounted ceramic (BR2)

- Shower area: walk-in 1,200 × 900 mm, rain shower head 200 mm + handheld, master also has 1,400 × 700 mm soaking nook

- Glass partition: 10 mm tempered, frameless, 1,800 mm tall

- Floor trap: 100 mm stainless trench (master) or 100 mm grating (BR2)

- Health faucet: ergonomic ABS, 1.5 m hose

- Geyser: 25 L instant electric, wall-mounted concealed

- Bathroom niche: 300 × 600 × 100 mm shower niche, LED strip-lit

- Exhaust fan: humidity-sensor 150 mm Ø, ducted to exterior

- Ventilation window: top-hung 600 × 450 mm with mosquito mesh

- Powder toilet: WC + counter-top basin + framed mirror, no shower

### Sheet 13 — Roof Plan @ 1:100

Show:

- Roof type: standing-seam metal sheet roof on insulated metal deck

- Roof slope: 5° single-slope from N to S (water flows south to RWH downpipes)

- Rainwater flow direction: arrows showing flow to gutters

- Gutters: continuous 150 mm PVC along south face, primed and painted

- Downpipes: 4 × 100 mm Ø downpipes routing to RWH tank

- Solar panel placement: 5–7 kWp PV array on south slope (between the U-shape arms), 13 × 410W panels

- Skylight location: above master toilet (1,200 × 800 mm)

- Roof overhangs: 1,200 mm on south face, 900 mm elsewhere

- Insulated roof detail: 75 mm PIR + acoustic underlay + standing-seam metal + high-SRI coating

- Drainage outlet points: 4 × downpipe inlets clearly marked

- Pergola / shade roof: over verandah (2,400 mm deep), timber-slat with 150 mm slat spacing

### Sheet 14 — Front (South) Elevation @ 1:100

Show:

- Premium forest container home facade — long horizontal silhouette

- Entry porch: pivot teak door, slate-tile threshold, 1,800 mm overhang

- Large sliding glass doors (3 × SD, 2,400 × 2,400 mm) on living + dining + kitchen

- Bamboo-batten cladding on container faces (vertical battens, 30 mm × 50 mm)

- Laterite stone base course at 300 mm plinth

- Standing-seam metal roof (mid-grey) with 1,200 mm overhang

- Pergola layer over verandah (timber slats)

- Landscape foreground: native canopy, mango/coconut trees in mid-distance

- Warm concealed lighting concept (LED strips at plinth, uplights at trees)

- Charred-timber feature wall on entry elevation (shou sugi ban)

### Sheet 15 — Rear (North) Elevation @ 1:100

Show:

- Forest-facing rear with bedrooms

- Master + BR2 large windows (W-04, W-07 — 1,800 × 1,500 mm each)

- Courtyard side visible through the U opening

- Privacy screen: timber slats at the rear, 1,800 mm tall, set 1,500 mm from elevation

- Large sliding glass doors (SD-04 master, SD-05 BR2) — 2,400 × 2,400 mm each

- Deep eaves on north face (900 mm)

- Standing-seam roof + solar PV visible from this angle

### Sheet 16 — Left (West) Side Elevation @ 1:100

Show:

- Master suite + master bath side wall treatment

- Windows: W-05 master (1,800 × 1,500), W-06 master bath (600 × 450 high-set)

- Bamboo-batten cladding

- Service access door (D-13) discreet

- Roof overhang line

- Cladding pattern detail (vertical bamboo battens with 5 mm reveal)

### Sheet 17 — Right (East) Side Elevation @ 1:100

Show:

- Kitchen + utility side

- Window W-03 kitchen (1,800 × 1,500), W-12 toilet ventilator (600 × 450)

- Outdoor shower option on master end (concealed enclosure 1,200 × 1,500)

- Service yard screening: 1,800 mm tall hedge or timber slat

- Charred-timber accent on utility module C-7

- Compost screen + RWH inlet visible

### Sheet 18 — Section A-A (longitudinal, N–S) @ 1:50

Cut through: Living room → glass spine → courtyard → deck → exterior

- FFL +600 mm above NGL

- Container floor build-up: helical pile + steel frame + composite deck + 50 mm PIR + screed + finish

- Wall build-up: container shell + 50 mm PIR + bamboo batten + 30 mm air gap

- Roof build-up: container roof + 75 mm PIR + acoustic underlay + standing-seam metal

- Ceiling height: 2,750 mm from FFL

- Roof height: 3,200 mm at high side, 2,950 mm at low side

- Deck level: −150 mm below FFL (step-down detail)

- Courtyard level: −300 mm below FFL (drainage)

- Show sliding glass door + verandah pergola in section

- Show 5° roof slope

### Sheet 19 — Section B-B (transverse, E–W) @ 1:50

Cut through: Bedroom → glass spine → bathroom → kitchen → utility

- Wet area details: tanking membrane + tile bed + finish

- Plumbing wall callout (drainage core)

- Roof insulation thickness

- Skylight detail above master toilet

- Show concealed AC indoor units in false ceiling

### Sheet 20 — Door and Window Schedule

| Code | Location | Size (mm) | Type | Material | Finish | Glass | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D-01 | Main entry | 1050 × 2400 | Pivot | Solid teak | Natural oil | — | Bronze pivot hinge, brass hardware |
| D-02 | Master BR | 900 × 2100 | Hinge | Engineered teak | Natural oil | — | Soft-close, brass |
| D-03 | BR2 | 900 × 2100 | Hinge | Engineered teak | Natural oil | — | Soft-close, brass |
| D-04 | Master toilet | 750 × 2100 | Hinge | WPC-core flush | Charred-timber veneer | — | Soft-close |
| D-05 | BR2 toilet | 750 × 2100 | Hinge | WPC-core flush | Charred-timber veneer | — | Soft-close |
| D-06 | Powder | 700 × 2100 | Hinge | WPC | Lime-wash | — | — |
| D-07 | Study/Lounge | 900 × 2100 | Sliding | Glass + teak frame | Frosted top + teak base | Frosted DGU | Top-hung |
| D-08 | Utility | 800 × 2100 | Hinge | WPC-core | Lime-wash | — | Mosquito mesh |
| D-09 | Store | 750 × 2100 | Hinge | WPC-core | Lime-wash | — | — |
| D-10 | Master WR | 800 × 2100 | Pocket | Engineered teak | Natural oil | — | Concealed track |
| D-11 | Service entry (utility yard) | 900 × 2100 | Hinge | Powder-coated steel + glass | RAL 7016 | Wired DGU | Lockable |
| D-12 | Courtyard side bedroom | 1200 × 2400 | Sliding | Aluminium frame + DGU | Natural anodised | Low-E DGU | Slide to courtyard |
| D-13 | Service utility | 800 × 2100 | Hinge | WPC | Lime-wash | — | — |
| D-14 | Verandah side | 900 × 2100 | Hinge | Teak | Natural oil | — | — |
| SD-01 | Living south | 2400 × 2400 | Triple-track sliding | Aluminium + DGU | Natural anodised | Low-E DGU | 3-panel, top + bottom track |
| SD-02 | Dining courtyard | 2400 × 2400 | Triple-track sliding | Aluminium + DGU | Natural anodised | Low-E DGU | Opens to courtyard |
| SD-03 | Kitchen east | 1800 × 2400 | Double-track sliding | Aluminium + DGU | Natural anodised | Low-E DGU | Opens to herbal garden |
| SD-04 | Master north | 2400 × 2400 | Double-track sliding | Aluminium + DGU | Natural anodised | Low-E DGU | Opens to private garden |
| SD-05 | BR2 north | 2400 × 2400 | Double-track sliding | Aluminium + DGU | Natural anodised | Low-E DGU | Opens to plantation |
| SD-06 | Study south | 1800 × 2400 | Double-track sliding | Aluminium + DGU | Natural anodised | Low-E DGU | Opens to verandah |
| W-01 | Living west | 1500 × 1200 | Top-hung | Aluminium + DGU | Natural anodised | Low-E DGU | Mosquito mesh |
| W-02 | Dining N | 1500 × 1200 | Fixed | Aluminium + DGU | Natural anodised | Low-E DGU | — |
| W-03 | Kitchen east | 1800 × 1500 | Slider | Aluminium + DGU | Natural anodised | Low-E DGU | Mosquito mesh |
| W-04 | Master east | 1800 × 1500 | Slider + fixed | Aluminium + DGU | Natural anodised | Low-E DGU | Mosquito mesh |
| W-05 | Master west | 1500 × 1200 | Slider | Aluminium + DGU | Natural anodised | Low-E DGU | Mosquito mesh |
| W-06 | Master bath | 600 × 450 | Top-hung | Aluminium + frosted DGU | Natural anodised | Frosted DGU | Mosquito mesh |
| W-07 | BR2 east | 1800 × 1500 | Slider | Aluminium + DGU | Natural anodised | Low-E DGU | Mosquito mesh |
| W-08 | BR2 bath | 600 × 450 | Top-hung | Aluminium + frosted DGU | Natural anodised | Frosted DGU | Mosquito mesh |
| W-09 | Study east | 1500 × 1200 | Top-hung | Aluminium + DGU | Natural anodised | Low-E DGU | Mosquito mesh |
| W-10 | Utility north | 1200 × 900 | Slider | Aluminium + DGU | Natural anodised | Clear DGU | Mosquito mesh + grill |
| W-11 | Store ventilator | 600 × 450 | Top-hung louvre | Aluminium | Natural anodised | — | Mosquito mesh |
| W-12 | Powder | 600 × 450 | Top-hung | Aluminium + frosted DGU | Natural anodised | Frosted DGU | Mosquito mesh |

### Sheet 21 — Material and Cladding Details @ 1:20 / 1:10

Show typical wall section details:

- External wall finish (top to bottom): bamboo batten cladding (30 × 50 mm) + 30 mm air gap + Tyvek membrane + 50 mm PIR insulation + container corrugated steel

- Stone plinth: 300 mm laterite stone + DPC + container floor edge

- Metal frame: container corner casting + welded MS frame + bolt detail

- Glass railing detail (deck edge): 10 mm tempered glass + stainless top rail + concealed shoe

- Deck material: 145 mm IPE planks on 60 mm WPC sleepers, 10 mm gap, hidden fixing

- Roof material: 76 mm standing-seam metal + 75 mm PIR + acoustic underlay + container roof deck

- Insulation: 50 mm walls + 75 mm roof (PIR / rockwool option)

- Bathroom tile finish: 600 × 300 mm natural slate + grout + sealer

- Kitchen counter: 20 mm quartz on plywood substrate

- Anti-rust coating: marine-grade epoxy primer + topcoat on all container shells

### Sheet 22 — Structural Concept Plan @ 1:100

Show:

- Modular frame line: container corner castings at 12,200 × 2,440 mm grid for 40 ft, 6,100 × 2,440 mm for 20 ft

- Foundation type: helical steel screw piles, 100 mm Ø, 3 m deep, 1,500 mm spacing along container edges

- Deck support: 50 mm Ø steel post grid at 1,800 mm spacing, hot-dip galvanised

- Roof support: container corner castings + welded steel header at module junctions

- Container / modular frame concept: ISO container shell as primary structure; cutouts reinforced with C-channel headers + flitch plates

- Structural engineer verification note: "All structural geometry, member sizing, foundation depth, and seismic / wind compliance to be certified by a licensed structural engineer per IS 875 (wind), IS 1893 (seismic), and IS 800 (steel)."

### Sheet 23 — Landscape Concept Plan @ 1:200

Show:

- Native tree preservation map (existing trees retained, marked X)

- Herbal garden (2,000 sq.ft) — tulsi, ashwagandha, brahmi, lemongrass, vetiver, mint, curry leaf

- Plantation zone (26,000 sq.ft) — buyer-selected: cashew / mango / coconut / spice / herbal / food forest

- Outdoor seating: 3 stone-bench clusters at view points along the walking trail

- Firepit (1,500 mm Ø raked-gravel pad with stone surround) at SE corner

- Pathway loop (900 mm wide stone-paver)

- Privacy planting belt (12 ft wide native canopy along boundary)

- Water feature: stone water channel in courtyard (3 × 1.2 m, 200 mm deep, recirculating)

- Stone seating along walking trail

- Forest buffer (native canopy + bamboo grove on west)

- Lighting points: 12 bollards along trail (5 W LED), 6 uplights on canopy trees, 4 path lights at courtyard

### Sheet 24 — Sustainability Plan @ 1:100 / NTS

Show as overlay + side panel:

- Solar-ready roof: 5–7 kWp PV array layout, inverter wall at utility, net-meter provision

- Rainwater harvesting: roof catchment → gutters → downpipes → 15,000 L tank → first-flush + multi-stage filter → underground sump

- Greywater reuse: kitchen + bath greywater → reed-bed wetland → drip irrigation

- Bio-septic / STP location: twin-chamber bio-septic at E utility zone, planted leach field

- Natural ventilation path: arrows showing SW monsoon air flow through glass spine + courtyard stack

- Cross ventilation: every habitable room has windows on at least 2 opposing faces

- Shading strategy: deep south overhang (1,200 mm) + west-elevation louvre screen + pergola layer

- Low-impact foundation: helical screw piles, zero concrete pour

- Local material usage: bamboo + laterite + lime plaster + Goa-sourced timber

- Waste segregation zone: 3-bin station at utility yard (wet / dry / recyclable)

- Drip irrigation: pressure-compensated grid, zoned by plantation profile

### Sheet 25 — Future Expansion Plan @ 1:100

Show dashed future-phase additions:

- Future third bedroom: 1 × 40 ft HC container attachable to BR2 wing

- Future spa / massage room: 1 × 20 ft module adjacent to master bath, with private outdoor shower

- Future pool deck expansion: 6 × 3 m lap pool + sun deck off the south verandah

- Future outdoor kitchen: 4 × 3 m pergola-roofed counter with sink and gas hob at SE deck

- Future staff / service room: 1 × 20 ft module at east service yard with separate entry + attached toilet

- Future rental lock-off option: BR2 + BR2 bath + study can be partitioned as a lock-off suite with separate entry from the side yard (for farm-stay rental mode)

### Sheet 26 — Area Statement (table)

| Group | Space | Area (sq.ft) |
| --- | --- | --- |
| Public | Entrance foyer | 100 |
| Public | Living room | 280 |
| Public | Dining area | 200 |
| Public | Open kitchen | 200 |
| Public sub-total |  | 780 |
| Private | Master bedroom | 300 |
| Private | Master wardrobe | 70 |
| Private | Master toilet | 140 |
| Private | Master private lounge nook | 110 |
| Private | Bedroom 2 | 280 |
| Private | Bedroom 2 toilet | 110 |
| Private sub-total |  | 1,010 |
| Family / Work | Study / family lounge | 140 |
| Family / Work | Powder toilet | 50 |
| Family sub-total |  | 190 |
| Service | Utility / laundry | 90 |
| Service | Store / pantry | 70 |
| Service sub-total |  | 160 |
| Outdoor enclosed | Glass-link corridors | 180 |
| Outdoor enclosed | Covered verandah | 200 |
| Outdoor enclosed | Courtyard transitions | 100 |
| Outdoor enclosed sub-total |  | 480 |
| TOTAL BUILT-UP |  | ~2,620 |
| Tolerance / wall thickness adjustment |  | −120 |
| TOTAL APPROX. BUILT-UP |  | ~2,500 |
| Plot area |  | 43,560 |

## 11. Door and window schedule (master)

See Sheet 20 above for the full schedule (14 doors + 6 sliding doors + 12 windows = 32 openings total). Glass openings comprise approximately 35% of external wall area — important for forest immersion. All sliding doors use low-E argon-filled DGU; all habitable-room windows have mosquito mesh + safety grille (concealed).

## 12. Electrical planning notes

- Load estimation: 12 kW connected load; 8 kW demand load; 1-phase / 3-phase to be confirmed with Goa Electricity Department.

- Solar integration: 5 kW hybrid inverter + 10 kWh battery + grid-tie; net-meter provision; load priority for refrigerator, water pump, kitchen, lighting.

- Earthing: dedicated copper earth pit (3 m deep) per IS 3043.

- Surge protection: Type 2 SPD at main DB.

- Cable runs: PVC conduit in walls; cable trays in ceiling void; routed through structural reinforcement gaps.

- Switch types: modular (Schneider / Legrand) for high-traffic; smart-home backbone (KNX or Loxone) provisioned for future automation.

- Outdoor lighting: weatherproof IP65 LED fittings; 2,700 K warm white only outside; 3,000 K interior bedrooms; 4,000 K kitchen.

- EV charging: 7.2 kW Type-2 socket at arrival forecourt; dedicated 32A circuit.

- MEP consultant verification required for final SLD and earthing design.

## 13. Plumbing planning notes

- Water source: borewell + RWH tank + municipal connection if available; 15,000 L underground sump + 5,000 L overhead.

- Booster pump: 1 HP pressure pump + hydropneumatic tank for uniform pressure.

- Hot water: instant electric geysers at point-of-use (25 L master, 25 L BR2, 10 L kitchen) + solar hot-water backup option.

- Pipe sizing: 20 mm CPVC mains, 15 mm fixture branches; 50 mm UPVC drainage, 100 mm soil; all per IS 4985.

- Drainage: separate stack for soil (WC) and greywater (basin, shower, kitchen).

- Greywater treatment: kitchen + bath greywater → grease trap → reed-bed wetland → drip irrigation; no greywater into bio-septic.

- Bio-septic: twin-chamber concrete tank, 1,500 L + 750 L, with planted leach field; designed per IS 2470.

- Rainwater harvesting: roof catchment area ~1,800 sq.ft, expected annual harvest ~80,000 L (Goa rainfall ~2,500 mm/yr).

- Outdoor taps: 4 hose bibs at corners + 1 at outdoor shower.

- MEP consultant verification required for final sizing, falls, and STP design.

## 14. Material specification notes

| Application | Material | Specification |
| --- | --- | --- |
| Foundation | Helical steel screw piles | 100 mm Ø, 3 m deep, hot-dip galvanised |
| Container shell | Recycled steel ISO container | 40 ft HC + 20 ft, marine-epoxy coating |
| Exterior cladding (primary, 70%) | Bamboo batten | 30 × 50 mm, treated, oiled, 30 mm air gap |
| Exterior cladding (accent) | Charred timber (shou sugi ban) | Goa-sourced teak / matti, deep-charred surface |
| Exterior cladding (panels) | Lime-plaster on insulated board | Off-white, smooth troweled |
| Plinth | Laterite stone | 300 mm height, dry-stacked + mortared |
| Roof | Standing-seam metal | 0.6 mm Galvalume, mid-grey high-SRI coating |
| Glazing | Low-E argon-filled DGU | 6-12-6 mm; thermal break aluminium frames |
| Interior floor (living, master) | Engineered oak | 180 × 1,800 × 14 mm, oiled |
| Interior floor (other) | Microcement | 4 mm seamless, sealed |
| Interior floor (wet zones) | Natural slate | Anti-skid, sealed |
| Outdoor decks | IPE / kapur hardwood | 145 mm planks, hidden fixing |
| Interior walls (general) | Lime-wash on insulated board | Soft white, breathable |
| Interior walls (feature) | Hand-finished plaster | Trowel-textured |
| Joinery | Oiled hardwood (teak, mango, jackwood) | Locally crafted |
| Kitchen counter | Quartz on plywood substrate | 20 mm, sealed edge |
| Hardware | Brass + oxidised bronze | Patinated finish, no chrome |
| Paint | Low-VOC water-based | Asian Paints / Berger eco line equivalent |
| Anti-rust coating (Goa coastal humidity) | Marine-grade epoxy primer + PU topcoat | 3-coat system on all container faces |

## 15. Landscape notes

- Native tree preservation: any existing tree > 200 mm girth to be retained; building footprint to be plotted around them.

- Privacy belt: 12 ft wide planting belt along plot boundary — Mangifera, Ficus, Anthocephalus, Terminalia at 3 m spacing.

- Herbal garden: 2,000 sq.ft raised beds adjacent to kitchen (E side); 600 mm high stone-edged beds; drip irrigation; partial shade.

- Plantation zone: 26,000 sq.ft buyer-selected profile; trees at 6–8 m spacing depending on species; mulched understory.

- Walking trail: 0.8 km loop, 900 mm wide laterite stone-paver; 3 stone-bench rest stops; lit by 12 LED bollards (5 W warm white).

- Butterfly garden: 200 sq.ft SE corner — Asclepias, Lantana, Stachytarpheta, Mussaenda.

- Bird-friendly planting: Plumeria, Ixora, Bauhinia, Cassia along central garden.

- Stone water channel: in courtyard, 3 × 1.2 m × 200 mm deep, recirculating with submerged pump, native fern margins.

- Firepit: 1,500 mm Ø raked-gravel pad with 4 stone bench seats; gas connection optional.

- Outdoor lighting: warm 2,700 K, IP65, dimmable; tree uplights, path bollards, courtyard wall washers, deck step lights.

- Permeability: zero impermeable surface in the plantation zone; driveway uses grass pavers; pathways use stone or gravel.

## 16. Sustainability notes

| Domain | Strategy |
| --- | --- |
| Solar | 5–7 kWp roof-mounted PV; 10 kWh battery; net-meter; load-priority on essentials |
| Rainwater | 15,000 L tank + recharge pit; first-flush + multi-stage filter; expected annual harvest ~80,000 L |
| Greywater | Reed-bed wetland → drip irrigation; no greywater to bio-septic |
| Bio-septic | Twin-chamber + planted leach field; no soak pit into groundwater |
| Cooling | Passive (courtyard stack effect + cross-vent + deep overhangs); split-DX backup |
| Insulation | 50 mm walls + 75 mm roof PIR; low-E DGU glazing |
| Composting | Twin-chamber rotary composter at utility yard |
| Lighting | 100% LED; 2,700/3,000/4,000 K zones; outdoor 2,700 K only |
| Foundation | Helical screw piles; zero concrete pour |
| Local materials | Bamboo, laterite, lime, Goa-sourced timber, native plantation |
| Anti-rust | Marine-grade epoxy on all container faces (Goa coastal humidity) |
| Drainage | Designed for Goa monsoon — 2,500 mm/year; permeable paving in 95% of outdoor surfaces |

## 17. Future expansion notes

The plan reserves a future expansion zone at the east boundary (dashed outline on Sheet 02 + Sheet 25). Options:

| Expansion | Module | Use case |
| --- | --- | --- |
| Third bedroom | 1 × 40 ft HC | Upgrade to 3BHK for growing family |
| Spa / massage room | 1 × 20 ft | Adjacent to master bath with private outdoor shower |
| Pool deck + lap pool | Hardscape | 6 × 3 m lap pool with sun deck |
| Outdoor kitchen | Pergola | 4 × 3 m counter, sink, gas hob — entertaining |
| Staff / caretaker room | 1 × 20 ft | Separate entry + attached toilet, eastern service yard |
| Rental lock-off | Partition only | BR2 wing partitioned with separate entry for farm-stay mode |

All future modules use the same container library and connect via extension of the existing glass-link spine. Foundations for future modules can be installed during Phase 1 to reduce future site disturbance.

## 18. CAD layer list

| Layer | Colour | Lineweight | Purpose |
| --- | --- | --- | --- |
| A-WALL | 7 white | 0.50 mm | Container shells + insulated walls |
| A-DOOR | 6 magenta | 0.25 mm | Door swings |
| A-WINDOW | 4 cyan | 0.25 mm | Window openings |
| A-FURN | 9 grey | 0.18 mm | Furniture indication |
| A-DIMS | 5 blue | 0.18 mm | Dimensions |
| A-TEXT | 7 white | 0.18 mm | Room names, annotation |
| A-HATCH | 252 | 0.13 mm | Material hatching |
| A-ROOF | 34 brown | 0.25 mm | Roof outline + slope arrows |
| A-DECK | 34 brown | 0.25 mm | Verandah + deck edges |
| A-COURTYARD | 41 sand | 0.20 mm | Courtyard slab + water channel |
| A-LANDSCAPE | 80 green | 0.18 mm | Plantation, lawn, walking trail |
| A-SITE | 7 white | 0.50 mm | Plot boundary |
| A-PARKING | 32 tan | 0.25 mm | Driveway, parking, arrival court |
| A-CONTAINER | 2 yellow | 0.50 mm | Container module outlines |
| A-PLANTATION | 84 | 0.15 mm | Plantation hatch infill (DASHDOT) |
| A-WATER | 14 cyan | 0.25 mm | RWH, septic, greywater, pool |
| A-UTILITY | 1 red | 0.25 mm | Solar, EV, gas, electrical mains |
| E-LIGHT | 3 green | 0.20 mm | Light points |
| E-POWER | 1 red | 0.20 mm | Power sockets |
| E-SWITCH | 6 magenta | 0.20 mm | Switches |
| E-DB | 1 red | 0.30 mm | Distribution board |
| P-WATER | 4 cyan | 0.20 mm | Water lines |
| P-DRAIN | 8 grey | 0.20 mm | Drainage |
| P-SANITARY | 21 brown | 0.20 mm | Sanitary fixtures |
| S-FRAME | 2 yellow | 0.40 mm | Structural frame |
| S-FOUNDATION | 1 red | 0.40 mm | Foundation lines |
| C-GRID | 9 grey | 0.13 mm (DASHED) | Grid lines |
| C-LEVEL | 9 grey | 0.13 mm | Level markers |

## 19. Annotation standards

Every drawing must include:

| Element | Standard |
| --- | --- |
| Drawing title | Cormorant Garamond / serif, 5 mm, bottom-right title block |
| Drawing number | Sans-serif, 3.5 mm, bottom-right |
| Scale | Sans-serif, 2.5 mm; bar scale visible |
| North arrow | RV custom block, top-right, 22 mm Ø |
| Room names | Sans-serif, 2.5 mm, uppercase + letter-spacing 0.5 mm |
| Dimensions | Aligned, 2.5 mm text, arrow 2.5 mm, extension line offset 1.5 mm |
| Levels | Triangle marker + FFL/SFL notation |
| Section markers | Bold arrow + section letter A-A, B-B |
| Elevation markers | Direction triangle + sheet reference |
| Door / window tags | Bubble tag with code (D-01, W-01, SD-01) |
| Container module labels | Bold C-1 to C-7 inside each module |
| Material notes | Leader line + bullet text, 2.5 mm |
| Revision box | Top-right or beside title block; rev / date / description / drawn by / checked by |
| Title block | Bottom-right, A1 sheet, includes project name, model, drawing title, scale, date, sheet no., revision, north arrow, drawn by, checked by, approved by |

## 20. AutoCAD drafter instructions

Step-by-step drafting workflow:

- Setup: open new drawing in metric units (mm); set Limits to 65,000 × 65,000 mm; set drawing scale 1:100 (model space), plotted A1.

- Layers: create all layers per Section 18 with assigned colours and lineweights.

- Title block: insert RV title-block template at all 26 sheet tabs.

- Plot boundary (Sheet 02): draw 63,400 × 63,400 mm rectangle on A-SITE.

- Container modules (Sheet 04, 05): draw 6 × (12,200 × 2,440 mm) rectangles for C-1 to C-6 and 1 × (6,100 × 2,440 mm) for C-7 on A-CONTAINER. Place in U-shape with 4,500 mm courtyard between the two long arms; orient long axis E-W for C-1 and C-2; orient long axis N-S for C-3 to C-6.

- Walls and partitions (Sheet 04): draw all walls on A-WALL; wall thickness 100 mm external (with insulation), 50 mm internal.

- Glass-link spine: draw curtain-wall corridor 2,200 mm wide × variable length on A-CONTAINER (hatched with glazing pattern).

- Courtyard: draw 8 × 5 m rectangle on A-COURTYARD; add stone water channel 3 × 1.2 m within.

- Doors and windows: insert RV-DOOR and RV-WINDOW blocks at every opening; tag with codes from Sheet 20 schedule.

- Furniture: import RV-FURN block library (sofa, dining, beds, wardrobes); place on A-FURN at 0.5 pt strokes.

- Plantation hatch (Sheet 02, 23): hatch plantation zone with ANSI31 @ 1,500 mm scale, rotation 45°, on A-PLANTATION.

- Utility symbols (Sheet 24): insert RV-UTIL block library (RWH, septic, composter, solar shed, EV).

- Dimensions (Sheet 07): use DIMSTYLE "RV-DIM"; text 2.5 mm; arrowhead 2.5 mm; extension lines offset 1.5 mm.

- Text styles: RV-TITLE (5 mm, font: serif), RV-NOTE (2.5 mm, font: sans), RV-ROOM (2.5 mm, uppercase, letter-spacing 0.5).

- Sections: cut Sections A-A and B-B; show on Sheets 18 and 19 at 1:50.

- Elevations: cut elevations from floor plan with VIEW commands; render with cladding hatch on A-HATCH.

- Save: DWG 2018 format; plot using RV-MONO.ctb for line weights; export PDF at A1 for each sheet.

- QA: verify every dimension closes (sum of internal dims = external); verify every door has a tag; verify every fixture has an electrical / plumbing point.

## 21. ASCII 2BHK floor plan (U-shape, top view)

[PLANTATION-FACING DECK + OUTDOOR DINING]

════════════════════════════════════════════════════════════════

(covered pergola + IPE timber deck)

────────────────────────────────────────────────────────────────

┌──────────────────┐  ┌──────────────────┐  ┌──────────────┐

│  C-1 ENTRY (100) │  │   C-2 DINING +   │  │  (south is   │

│  + LIVING (280)  │──│   OPEN KITCHEN   │──│   the open   │

│                  │  │   (200 + 200)    │  │   side of U) │

│  (sliding glass  │  │                  │  │              │

│   south face)    │  │                  │  │              │

└────────┬─────────┘  └────────┬─────────┘  └──────────────┘

[Link 1]              [Link 2]              ↓ to east yard

┌────────┴──────────────────────┴───────────────────┐

│                                                    │

│   [ENTRY]      ┌──────────────────────────┐       │

│   FOYER  ──────│  COURTYARD (open-to-sky) │       │   GLASS-LINK

│                │  8 × 5 m · stone water    │  [L4] │   SPINE

│   GLASS-LINK   │  channel · fern planting  │       │

│     SPINE      │  · single Plumeria tree   │       │

│                └──────────────────────────┘       │

└────────┬─────────────────┬────────────────┬───────┘

[Link 3]           [Link 5]         [Link 6]

┌────────┴────────┐  ┌────┴───────┐  ┌────┴────────┐

│  C-3 MASTER     │  │ C-4 M.WR   │  │ C-5 BR 2    │

│  BEDROOM (300)  │  │ + M.BATH   │  │ (280)       │

│                 │  │ + M.LOUNGE │  │             │

│                 │  │ (70+140+   │  │             │

│                 │  │ 110)       │  │             │

└────────┬────────┘  └────────────┘  └─────┬───────┘

[Link 7]                            [via Link 6]

┌────┴────────┐

│ C-6 BR2     │

│ BATH (110)  │

│ + STUDY/    │

│ FAM.LOUNGE  │

│ (140)       │

│ + POWDER    │

│ (50)        │

└─────┬───────┘

service spine ↓

┌────┴────────┐

│ C-7 (20 ft) │

│ UTILITY (90)│

│ + STORE (70)│

└─────────────┘

[COVERED FOREST VERANDAH wraps the south + courtyard inner face]

──────────────────────────────────────────────────────────────

(bamboo sun screen + pergola layer · 200 sq.ft)

## 22. ASCII 1-acre site plan (central placement)

+──────────────────────────────────────────────────────────────+

│  LANDSCAPE BUFFER — privacy trees · biodiversity corridor    │

│                                                               │

│   +──── PLANTATION (buyer-selected) ──────────────────────+  │

│   │  cashew · mango · coconut · spice · herbal · food forest │

│   │                                                       │  │

│   │           ┌── 2BHK U-SHAPE PAVILION ──────┐           │  │

│   │           │  [LIVING][DINING/KITCHEN]      │           │  │

│   │           │  [glass-link spine + 8x5m     │           │  │

│   │           │   COURTYARD with stone        │           │  │

│   │           │   water channel + Plumeria]    │           │  │

│   │           │  [MASTER][M.BATH][BR2][BR2-B] │           │  │

│   │           │  [STUDY/FAM][C-7 UTILITY]     │           │  │

│   │           │      verandah · deck          │           │  │

│   │           └────────────────────────────────┘           │  │

│   │                                                       │  │

│   │       optional plunge pool · firepit · yoga lawn     │  │

│   │       herbal garden + drip grid                       │  │

│   │       meditation pad                                  │  │

│   │                                                       │  │

│   │   ┌── RWH 15k L ──┐  ┌── greywater reed bed ───┐    │  │

│   │   └───────────────┘  └──────────────────────────┘    │  │

│   │   ┌── bio-septic ──┐  ┌── compost twin ──┐           │  │

│   │   └────────────────┘  └─────────────────┘            │  │

│   │   ┌── future expansion zone (dashed) ──┐             │  │

│   │   └────────────────────────────────────┘             │  │

│   +─────────────────────────────────────────────────────+   │

│                                                               │

│   ::driveway::  [arrival court + 2-car parking]              │

│   ════GATE══════════════════════ access road ════════════════│

+──────────────────────────────────────────────────────────────+

N ▲

scale: 1 acre = ~208 × 208 ft

(central placement option B)

## 23. SVG drawing prompt — premium 2D floor plan

Generate a 2400 × 1600 px presentation-grade SVG combining the 1-acre site

plan with the embedded ground-level 2BHK U-shape floor plan, for the 2BHK

Compact Modular Eco Estate at Reserva Verde Goa.

Layout: U-shape pavilion plan formed by 6 × 40 ft high-cube containers +

1 × 20 ft service module around an 8 × 5 m central courtyard with stone

water channel and single Plumeria tree. Central placement on a 208 × 208 ft

square plot.

Palette (premium forest eco):

- background:       #FBF8F1 (ivory)

- container walls:  #1F3A2E (forest green outlines, 1.6 pt)

- glazing/links:    #B08A3E (muted gold, hairline)

- interior fill:    #F4EDDD (beige) for living/bedrooms

- service fill:     #E8DFC4 (warm beige) for utility/wet

- landscape zones:  #2E5944 (deep plantation green) with #3F7257 scatter

- water:            #7FA9C9 (soft slate-blue)

- charcoal labels:  #2A2A2A

SHOW:

- 1-acre boundary with 12 ft landscape buffer

- Central placement of the U-shape home

- Driveway from south gate to 2-car forecourt

- All 7 container outlines with IDs C-1 to C-7 + dimensions (ft + m)

- Room names: ENTRY FOYER, LIVING, DINING, KITCHEN, MASTER BR, MASTER WR,

MASTER BATH, MASTER LOUNGE, BR2, BR2 BATH, STUDY/FAMILY LOUNGE, POWDER,

UTILITY, STORE, COURTYARD, VERANDAH, DECK

- Door swings labelled D-01 to D-14, sliding doors SD-01 to SD-06

- Windows W-01 to W-12 as gapped wall segments with sill line + sizes

- Bathroom fixtures (WC, basin, shower tray, soaking nook) as top-view glyphs

- Kitchen counters with sink + gas hob + breakfast island

- Furniture in light grey 0.5 pt strokes (sofa, dining table+6, beds, wardrobes,

study desk + bookshelves)

- Courtyard with stone water-channel rectangle + canopy-tree disc

- Optional plunge pool 6 × 2.5 m in outdoor lifestyle zone

- Covered verandah + outdoor dining pergola hatched lightly

- Glass-link spine in hairline gold

- Plantation hatching at 2500 mm scale, rotated 45°

- RWH tank NE, recharge pit, bio-septic + reed bed E, composter, solar shed SW

- Walking trail loop (dashed gold) through plantation

- Future expansion outline (dashed) at E boundary

- North arrow top-right, scale bar bottom-left (0–10–20–40 ft)

- Legend bottom-right

- Title block top-right: "RESERVA VERDE GOA · 2BHK · 2500 SQ.FT · 1 ACRE

ESTATE · U-SHAPE COURTYARD · SITE + FLOOR PLAN · SCALE 1:100/1:200"

- Hairline 1-ft grid 30% opacity

- Strokes: 1.6 pt container walls; 0.6 pt internal partitions; 0.4 pt furniture;

hairline grid

Strict top-view orthographic — no 3D shading, no perspective.

## 24. SketchUp modelling prompt

Open a new SketchUp model — Architectural Design Meters template.

PROJECT GEOREFERENCING:

- Geolocate to Goa, India (~15.30° N, 73.98° E)

- Shadow study: 17:30 IST, March equinox

GROUND PLANE:

- 63.4 × 63.4 m flat slab on layer "00-GROUND"

- Subtle 2% slope NW to SE

COMPONENTS (build once, place as instances):

- CNT-40HC: 12.19 × 2.44 × 2.90 m, chamfered corners, corrugated side faces,

corner castings, thin roof cap

- CNT-20HC: 6.10 × 2.44 × 2.90 m same treatment

- LNK-GLASS-2200: 2.2 m wide × 3.0 m tall curtain-wall corridor with

1,200 mm mullions

U-SHAPE PLACEMENT (X, Y in metres; rot in degrees):

SOUTH ARM (public):

- C-1 Entry + Living:  X=-8   Y=+5   rot=0   (long axis E-W)

- C-2 Dining + Kit:    X=+6   Y=+5   rot=0

EAST ARM (private right):

- C-5 BR2:             X=+11  Y=-4   rot=90  (long axis N-S)

- C-6 BR2 Bath + Std:  X=+11  Y=-12  rot=90

- C-7 Utility (20 ft): X=+11  Y=-17  rot=90

WEST ARM (private left):

- C-3 Master:          X=-13  Y=-4   rot=90

- C-4 M.WR + M.Bath:   X=-13  Y=-12  rot=90

CENTRAL COURTYARD:

- 8 × 5 m open-to-sky court between the three arms

- Stone water channel 3 × 1.2 m down centre (water-plane component)

- Single Plumeria tree component at one end

- Raked-gravel surround

ROOF: 5° single-slope standing-seam metal, 1,200 mm south overhang, 900 mm

elsewhere; pergola timber-slat layer over verandah; discreet 13 × 410 W PV

panels on south slope.

MATERIALS:

- MAT-bamboo  (rainscreen 70%)

- MAT-plaster-OW (lime, off-white)

- MAT-charred  (entry feature wall, shou sugi ban)

- MAT-laterite (300 mm plinth base course)

- MAT-metal-MG (mid-grey standing-seam roof)

- MAT-IPE     (timber deck on verandah)

LARGE GLASS OPENINGS:

- Sliding glass walls 2,400 × 2,400 mm on south face of C-1 (living)

- C-2 dining-to-courtyard 2,400 × 2,400 mm

- Master east face full-height glazing 2,400 × 2,400 mm

- BR2 east face 2,400 × 2,400 mm

COVERED VERANDAH wraps the south + courtyard inner face — 2.4 m deep × 16 m

long with pergola roof.

OPTIONAL PLUNGE POOL: 6 × 2.5 m × 0.6 m deep recessed pool off the south

deck with stone deck surround.

DRIVEWAY + PARKING: 4 m wide gravel driveway from south gate to a 6 × 8 m

forecourt with 2-car parking + EV charge.

FOREST BUFFER: place 35+ tree components along plot boundary (Mangifera

indica, Cocos nucifera, Anacardium occidentale from 3D Warehouse).

UTILITY ZONE: 2,500 mm Ø RWH tank component at NE, 2 × 2 m solar inverter

shed at SW, 1.2 × 0.8 m composter twin, 3 × 1.5 m greywater reed bed (E).

PATHWAYS: 0.9 m wide stone-paver trail looping the plot; secondary herbal

garden path off the kitchen.

HERBAL GARDEN: 6 × 4 m raised bed adjacent to kitchen.

EXPORT: render with Enscape or V-Ray Sun System; output 4,000 × 2,250 px

aerial isometric (golden hour); plus 3-view sheet (plan, south elevation,

east elevation). Save as RV_2BHK_v01.skp.

## 25. 3D render prompt

A premium ground-level 2BHK forest container home, set within a private

1-acre Reserva Verde Goa estate. Six 40-ft and one 20-ft high-cube shipping

containers arranged in an elegant U-shape pavilion plan with an 8 × 5 m

central open-to-sky courtyard at the heart — strictly single-storey,

no stacking, reading as a contemporary tropical eco-pavilion in the Aman /

Six Senses tradition, not a container house.

Natural materials: bamboo-batten rainscreen cladding, charred-timber

(shou sugi ban) feature wall on entry elevation, laterite stone base course

at 300 mm plinth, lime-plaster end panels, standing-seam metal roof with

discreet flush PV array. Large floor-to-ceiling glass sliders open onto

the courtyard and the south verandah.

Central open-to-sky courtyard with a stone water channel, native fern

planting, and a single Plumeria canopy tree. Wrap-around forest verandah

(2.4 m deep) with bamboo sun-screen and pergola roof. Optional plunge pool

6 × 2.5 m on the south deck.

Surrounding 1-acre plantation: cashew, mango, coconut, and a herbal-garden

strip alongside the kitchen wing. Gravel driveway sweeps from a low laterite-

stone gate to a 2-car porch. Native canopy buffer along the boundary. A

15,000 L RWH tank screened by tall ornamental grass at NE.

Lighting: soft golden hour, warm low sun raking across the bamboo cladding,

soft mist hovering over the plantation. Drone-level aerial perspective from

south-west, 40° elevation, 24 mm equivalent focal length for natural

perspective, no fisheye distortion.

Atmosphere: luxury eco-resort estate, calm, premium, eco-sensitive, intimate

2BHK scale. No people, no clutter, no logos, no signage.

Style cues: high-end real estate brochure, contemporary tropical luxury,

architectural visualization at Hasselblad 50 mm character. Photorealistic

8 K octane render with subtle volumetric god rays through the canopy.

## 26. Premium upgrades for ₹2000 Cr positioning

These are optional buyer-upgrade items that elevate the 2BHK to flagship-tier finish, suitable for the brand's ₹2000 Cr masterplan positioning:

| Upgrade | Description | Approx cost band |
| --- | --- | --- |
| Brushed brass hardware | All door / cabinet hardware upgraded to brushed brass (no chrome) | ₹2–3 lakh |
| Hand-finished plaster on living wall | Tadelakt-style lime plaster, hand-troweled, sealed | ₹1.5–2.5 lakh |
| Quartzite master vanity | Single-slab quartzite vanity top + stone basin | ₹1.5–2 lakh |
| Soaking tub (master) | Freestanding stone or copper tub in master bath | ₹3–5 lakh |
| Outdoor shower | Concealed stone-enclosed outdoor shower off master | ₹2–3 lakh |
| Wine fridge + bar | Built-in wine fridge + bar cabinet in dining wing | ₹2–3 lakh |
| Smart-home automation | KNX or Loxone backbone — lighting, AC, blinds, security | ₹4–6 lakh |
| Premium audio system | Sonos/Bose ceiling speakers in living + master | ₹2–3 lakh |
| Plunge pool | 6 × 2.5 m stone-clad plunge pool with heater | ₹6–8 lakh |
| Indoor garden niche | Built-in indoor planter wall in foyer or courtyard side | ₹1–2 lakh |
| Cedar sauna | Compact 2-person cedar sauna in master bath wing | ₹3–4 lakh |
| EV charge wall box | Wall-mounted 7.2 kW EV charger with smart metering | ₹50k–1 lakh |
| Skylight (master toilet) | Operable tempered-glass skylight + auto-rain sensor | ₹1.5–2 lakh |
| Outdoor kitchen | Pergola-roofed counter with sink, gas hob, fridge | ₹3–4 lakh |
| Firepit + stone seating | Gas firepit pad + 4 stone bench seats | ₹1–1.5 lakh |
| Acoustic study insulation | Cork acoustic walls + sound-tested glazing | ₹1–1.5 lakh |
| Rainwater chain | Architectural copper rainwater chain at front overhang | ₹50k–1 lakh |
| Curated art package | 5–8 framed art pieces by Goan / Indian artists | ₹3–5 lakh |
| Linen + interior package | Premium soft furnishings + curated decor | ₹4–6 lakh |
| Lighting design | Full custom lighting design by a specialist | ₹2–3 lakh |

Total premium upgrade band: ₹40–65 lakh above the base build.

## 27. Compliance disclaimer

This dossier is a concept-stage design document for planning, presentation, and preliminary architectural discussion only. Final design, dimensions, structural details, MEP, environmental compliance, permissions, and construction feasibility must be verified by licensed architects, structural engineers, surveyors, MEP consultants, environmental consultants, and relevant government authorities.

Specifically:

- Final drawings must be checked as per local Goa building rules (TCP, GCDA, Panchayat / Municipality as applicable).

- Structural design must be certified by a licensed structural engineer per IS 875 (wind), IS 1893 (seismic), IS 800 (steel), IS 13920 (RC detailing if applicable).

- Plumbing and electrical must be verified by a licensed MEP consultant per IS 1172 (water supply), IS 2470 (sewage), IS 732 (electrical).

- Septic / STP design must follow local environmental norms and CGWB norms for greywater.

- Tree cutting should be avoided wherever possible; tree census required before site clearance.

- Rainwater and drainage must be designed for Goa heavy monsoon (annual ~2,500 mm).

- Fire safety and access must be considered if used as rental or retreat product (NBC 2016 fire-safety guidelines).

- Forest / wildlife / environmental permissions, if applicable, must be verified by consultants and authorities.

- No yield, return, or appreciation projections are made or implied in this document.

## 28. Final checklist before CAD drafting

Hand this to the drafter. They should not start the package until every box below is ticked.

### Pre-drafting confirmation

- [ ] Confirm plot survey is available with exact dimensions and contour map

- [ ] Confirm soil percolation test result (drives recharge pit + leach field sizing)

- [ ] Confirm existing tree census on plot (drives building footprint placement)

- [ ] Confirm road access width and gate position

- [ ] Confirm true north for site orientation

- [ ] Confirm prevailing wind direction (Goa SW monsoon)

- [ ] Confirm view corridors (which direction has the best plantation/valley view)

- [ ] Confirm buyer's plantation profile selection (cashew / mango / coconut / spice / herbal / food forest)

- [ ] Confirm placement option (A / B / C / D — default B central)

- [ ] Confirm plunge pool option (yes / no)

- [ ] Confirm staff/caretaker room option (yes / no — adds 1 × 20 ft module)

- [ ] Confirm rental-readiness scope (off / light farm-stay / full retreat)

- [ ] Confirm interior package (eco-essential / premium / signature)

- [ ] Confirm premium upgrade selections from Section 26

### Drafting workflow

- [ ] Set up AutoCAD with metric units (mm), 1:100 scale, A1 sheet template

- [ ] Create all 28 layers per Section 18

- [ ] Insert RV title-block template on all 26 sheets

- [ ] Draft Sheets 01 to 26 in the listed order

- [ ] Cross-reference dimensions across plan / elevation / section

- [ ] Verify door / window schedule matches plan tags

- [ ] Verify electrical layout matches load estimation

- [ ] Verify plumbing layout matches fixture schedule

- [ ] Verify all dimensions close (sum of internal = external)

- [ ] Generate PDFs at A1 for each sheet using RV-MONO.ctb plot style

- [ ] Save DWG 2018 master file + revision history

### Pre-handover checks

- [ ] All sheets have title block, north arrow, scale bar

- [ ] All dimensions are double-checked

- [ ] All hatches close cleanly

- [ ] All annotations are spell-checked

- [ ] Door / window schedule matches placed openings

- [ ] Furniture schedule matches placed furniture blocks

- [ ] Structural engineer has reviewed Sheets 18, 19, 22

- [ ] MEP consultant has reviewed Sheets 10, 11, 12, 24

- [ ] Landscape consultant has reviewed Sheet 23

- [ ] Sustainability consultant has reviewed Sheet 24

- [ ] Compliance disclaimer is present on Cover Sheet

- [ ] Drawing index on Cover Sheet matches actual sheet list

- [ ] All revision boxes are updated

All figures are concept-stage and subject to final architectural planning, statutory approvals, and Reserva Verde Goa eco-sensitive development guidelines. No yield, return, or appreciation projections are made or implied.

Reserva Verde Goa · Bluechip Global Agro · 2BHK CAD Brief v1.0